namespace Feel

{

/** \page FictitiousDomainPenalisationParticle Penalty method to simulate particle flow in capillaries

\tableofcontents

\section FictitiousDomainPenalisationParticle_Intro Introduction

We propose a somewhat simple numerical method albeit used

conjointly with physical experiments to understand particle flows

in capillaries.  Studies of flows in micro channels have been

multiplied since several years because of the goal to create cheap

and efficient lab-on-chips. An application that would like to

achieve many groups with a good quality is the sorting of

particles on lab-on-chips. For example sorting healthy / sick red

blood cells. One of the building blocs of such microfluidic system

is the splitting of a channel into two daughters channels. All the

mechanisms involving the spreading of particles in such a system

are not completely understood. It has been seen for example that,

when a suspension of particles pass through a bifurcation having

different flow rates at outlet, the branch having the higher flow

rate see its volume fraction (concentration of particles)

increase. This phenomenon is known as the \textit{Zweifach - Fung

effect} \cite{YuanCheng197334}.

In \cite DOYEUX2011a, the authors used \c FreeFem++ to run a 2D

simulation of a microfluidic bifurcation in which a rigid and circular particle

is put in the inlet channel and goes to one of the outlet channels. The choice

of the particle to go to the higher or the lower flow rate channel depends on many

parameters which have been studied in the paper such as the flow rate ratio

between the two branches, the size of the particle, the size of the outlet

channels or even the angle between the inlet and the outlet channels. The

simulations were compared with experiments and showed a good agreement. These

simulations and experiments shown an attracting hydrodynamical force toward the

lower flow rate channel which was unknown until there. Moreover the increase in

concentration in the highest flow rate branch has been explained by a

geometrical effect of the distribution of particles in the inlet channel which

was a good step forward in the understanding of the \e Zweifach-Fung \c effect.

\section FictitiousDomainPenalisationParticle_Model Model and numerical method

We propose to implement the method used in \cite DOYEUX2011a and have both 2D

and 3D setups which is readily the case using \c Feel++. Moreover, \c Feel++

allows us to run the same simulation in 3D without re-coding.

The size of the channel is very small (typically

\f$\unit{60}{\micro\meter}\f$), so that we can assume that we are in a low Reynolds

number regime  (\f$Re=10^{-1}\f$ in \cite DOYEUX2011a). The particles are rigid

and spherical (circular in 2D) and for the sake of simplicity, we consider only one

particle in the bifurcation. The extension  to many particles can be done

with no special care since the particles do not touch. This assumption is valid

since we are interested in dilute suspension and in this case the hydrodynamic

force is generally sufficient to prevent contacts.

Let's call \f$\Omega_f\f$ the fluid domain, \f$B\f$ the particle domain and \f$\Omega = \Omega_f \cup

B\f$ the whole domain. As explained above, the fluid is assumed to be governed by Stokes equations

\f{eqnarray}{

- 2 \nu \nabla \cdot \bm{D(u)} + \nabla p &=& 0 \:  \text{ in } \Omega_f, \label{eq:stokes_1}\\

\nabla \cdot \bm{u} &=& 0 \: \text{ in } \Omega_f, \label{eq:stokes_2} \\

\bm{u} &=& \bm{f} \:  \text{ on } \partial \Omega_f\setminus\partial B, \label{eq:stokes_3}

\f}

where : \f$\bm{u}\f$ is the velocity of the fluid,

\f$\bm{D(u)} =  \dfrac{\nabla \bm{u} + \nabla \bm{u}^T}{2}\f$

the deformation tensor, \f$p\f$ is the pressure of the fluid and

\f$\bm{f}\f$ is a given function related to the boundary conditions on

\f$\partial\Omega\f$ (those on \f$\partial B\f$ will be treated later by

penalty). Moreover, we impose that the mean of the pressure is zero by adding a

Lagrange multiplier \f$\lambda\f$. This approach allows us to use direct solvers

such as UMFPack, MUMPS or PASTIX otherwise only iterative solvers would have

been accessible. Thus, the variational formulation reads :

Find \f$(\bm{u}, p, \lambda) \in H^1(\Omega_f)^2\times L^2_0(\Omega_f)\times\mathbb{R}\f$

such that \f$\forall (\bm{v}, q, \mu) \in H^1(\Omega_f)^2\times L^2_0(\Omega_f)\times\mathbb{R}\f$ :

\f{eqnarray}

2 \nu \int_{\Omega_f} \bm{D(u)} : \bm{D(v)} - \int_{\Omega_f} p \nabla \cdot

\bm{v} &&\nonumber\\

+ \int_{\Omega_f} \lambda q &=& 0, \label{eq:varstokes_1}\\

\int_{\Omega_f} q \nabla \cdot \bm{u} &=& 0, \label{eq:varstokes_2} \\

\int_{\Omega_f} \mu p &=& 0, \label{eq:varstokes_4}\\

\bm{u} &=& \bm{f} \: \text{ on } \partial \Omega. \label{eq:varstokes_3}\nonumber

\f}

and the corresponding \Feel code is given by the listing \ref{listing_stokes}.

The particle is taken into account following \cite Janela2005 in

which, one imposes a rigid-body constraint by a penalty

method. From the physics point of view, this method consists to

consider the particle as a part of the fluid with a ``huge

viscosity'' and constrained to have a rigid-body motion. Thus,

instead of integrating in the fluid domain \f$\Omega_f\f$, we

rewrite our variational formulation in the whole domain

\f$\Omega\f$ while taking into account the following constraint

\f[

\bm{D(u)} = 0, \: \text{ in } B.

\label{eq:rigidbody}

\f]

This rigid-body constraint \eqref{eq:rigidbody} is added to equation

\eqref{eq:varstokes_1} using a small enough penalty coefficient

\f$\varepsilon\f$.  Finally the new variational formulation reads

Find \f$(\bm{u},p, \lambda) \in H^1(\Omega)^2 \times L^2_0(\Omega) \times \mathbb{R}\f$ such

that \f$\forall (\bm{v}, q, \mu) \in H^1(\Omega)^2 \times L^2_0(\Omega) \times \mathbb{R}\f$ :

\f{eqnarray}{

2 \nu \int_{\Omega} \bm{D(u)} : \bm{D(v)} - \int_{\Omega} p \nabla \cdot \bm{v} && \nonumber\\

+\int_{\Omega} \lambda q + \frac{2}{\varepsilon} \int_B \chi ( \bm{D(u)} : \bm{D(v)} )  &=& 0, \label{eq:varstokespenal_1}\\

\int_{\Omega} q \nabla \cdot \bm{u} &=& 0, \label{eq:varstokespenal_2} \\

\int_{\Omega} \mu p &=& 0, \label{eq:varstokespenal_4}\\

\bm{u} &=& \bm{f} \: \text{ on } \partial \Omega. \label{eq:varstokespenal_3}\nonumber

\f}

Note that the integral over \f$B\f$ is performed by integrating over \f$\Omega\f$ while

multiplying by the characteristic function \f$\chi\f$ (see listing

\ref{listing_penalty}). In practice, only the penalty term changes during time

and has to be reassembled (e.g. the last term in equation

\eqref{eq:varstokespenal_1}). Thus, only the penalty term is added to the whole

problem at each iteration. Moreover, this term does vanishes on most

elements of the mesh and instead of integrating the penalty term over all the

elements and multiplying by \f$\chi\f$, we only integrate on those crossed by

\f$\chi\f$ using a special feature \c broken() of \c integrate()

to localize these elements, see section~\ref Integrals.

Finally, we introduce the time step \f$\Delta t\f$ and we denote by

\f$(\bm{u}_n,p_n,\lambda_n)\f$ the solution of the system

\eqref{eq:varstokespenal_1}-\eqref{eq:varstokespenal_2}-\eqref{eq:varstokespenal_4}

at time \f$t_n=n\Delta t\f$.  The velocity of the particle denoted by

\f$\bm{V}_n\f$ is then calculated by

the relation \eqref{eq:velocitypart} and its position \f$\bm{X}_n\f$ by the relation

\eqref{eq:positionpart}.

\f{eqnarray}{

\bm{V}_n &=& \frac{1}{\int_{\Omega} \chi} \int_{\Omega} \chi \, \bm{u}_n, \label{eq:velocitypart}\\

\bm{X}_{n+1} &=& \bm{X}_{n} + \Delta t \bm{V}_n, \label{eq:positionpart}

\f}

\section FictitiousDomainPenalisationParticle_Results Numerical Results

Figure shows a 2D simulation of a particle entering in the low

flow rate branch while figures and display the results of the 3D

simulation.

<center>

<table border=0px>

<tr>

<td style="width:30%">\image html bift_001 "t=0.01" width="100%"</td>

<td style="width:30%">\image html bift_120 "t=1.20" width="100%"</td>

<td style="width:30%">\image html bift_260 "t=2.60" width="100%"</td>

</tr>

<tr>

<td><center>2D simulation of a particle entering in the low flow rate channel.</center></td>

</tr>

<table>

<center>

<table border=0px>

<tr>

<td width="15%">\image html bif_vecteurs.png</td>

</tr>

<tr>

<td><center>3D simulation of the bifurcation problem. The velocity vectors are plotted and colored by the velocity magnitude. </center></td>

</tr>

</table>

</center>

*/

}